/*

* Channel 1: Count total amount of used water.

* Channel 2: Measures water pressure in pipe.

* Channel 3: Measures amount of water in litres that flows in an hour.

*/

//Pins

//--------------------

#define PRESSURE_SENSOR A3

#define PULSES_METER A2

//--------------------

//calculation constants

//--------------------

#define MAX_VOLTAGE 499UL

#define MAX_BAR 1000000UL

#define PULSES_PER_LITER 477UL

#define FLOW_SECONDS_INTERVAL 15UL // interval in which we update water flow speed

#define SECONDS_IN_HOUR 3600UL

//--------------------

#define WATER_METER_DELTA 2

#define PRESSURE_VALUE_DELTA 10000

#define WATER_FLOW_DELTA 500

dword pressure_value;

dword liters_value;

bool pulse_trigger = false;

byte timer_pulses = 0;

dword total_pulses = 0;

dword water_flow = 0;

dword last_pulses_time_check = 0;

dword last_reported_water_meter = 0;

dword last_reported_pressure_value = 0;

dword last_reported_water_flow_value = 0;

dword pulses_last_check = 0;

ZUNO_SETUP_CHANNELS(

ZUNO_METER(ZUNO_METER_TYPE_WATER, METER_RESET_ENABLE, ZUNO_METER_WATER_SCALE_METERS3, METER_SIZE_FOUR_BYTES, METER_PRECISION_THREE_DECIMALS, water_getter, water_resetter),

ZUNO_SENSOR_MULTILEVEL(ZUNO_SENSOR_MULTILEVEL_TYPE_BAROMETRIC_PRESSURE, SENSOR_MULTILEVEL_SCALE_KILOPASCAL, SENSOR_MULTILEVEL_SIZE_FOUR_BYTES, 3, pressure_getter),

ZUNO_SENSOR_MULTILEVEL(ZUNO_SENSOR_MULTILEVEL_TYPE_GENERAL_PURPOSE_VALUE, SENSOR_MULTILEVEL_SCALE_DIMENSIONLESS_VALUE, SENSOR_MULTILEVEL_SIZE_FOUR_BYTES, SENSOR_MULTILEVEL_PRECISION_ZERO_DECIMALS, water_flow_getter)

);

ZUNO_SETUP_ISR_1MSTIMER(m1ms_handler);

void setup() {

pinMode(PRESSURE_SENSOR, INPUT);

pinMode(PULSES_METER, INPUT);

}

void loop() {

noInterrupts();

if (timer_pulses > 0) {

total_pulses += timer_pulses; //counting pulses for both of first and third channels.

timer_pulses = 0;

}

interrupts();

//Channel 1:

liters_value = total_pulses / PULSES_PER_LITER;

//Channel 2:

pressure_value = analogRead(PRESSURE_SENSOR) * MAX_BAR / 821;    //converting analog signal from pressure sensor into pascals.

//Channel 3:

if ((millis() - last_pulses_time_check) > (FLOW_SECONDS_INTERVAL * 1000)) { // counting water flow

last_pulses_time_check = millis();

water_flow = ((total_pulses - pulses_last_check) * (SECONDS_IN_HOUR / FLOW_SECONDS_INTERVAL)) / PULSES_PER_LITER;

pulses_last_check = total_pulses;

}

//sending reports if it is needed. Checking the histeresis for all the channels:

if (abs(liters_value - last_reported_water_meter) > WATER_METER_DELTA){

zunoSendReport(1);

last_reported_water_meter = liters_value;

}

if (abs(pressure_value - last_reported_pressure_value) > PRESSURE_VALUE_DELTA){

zunoSendReport(2);

last_reported_pressure_value = pressure_value;

}

if (abs(water_flow - last_reported_water_flow_value) > WATER_FLOW_DELTA){

zunoSendReport(3);

last_reported_water_flow_value = water_flow;

}

}

// Getters & Setters

// ------------------------------

dword pressure_getter() {

return pressure_value;

}

dword water_getter() {

return liters_value;

}

void water_resetter() {

liters_value = 0;

total_pulses = 0;

}

dword water_flow_getter(){

return water_flow;

}

// ------------------------------

//This is Timer handler that triggers every 1 ms.

void m1ms_handler(){

if (!digitalRead(PULSES_METER)){

if (!pulse_trigger){

pulse_trigger = true;

timer_pulses++;

}

}

else pulse_trigger = false;

}